VS-RK3399 Specification

Content

Cover……………………………………………………1

Content………………………………………………….2

1、Specification …………………………………….. . 3

2、Product Picture……………………………………..4

3、Ports parameter ……………………………….........5

4、Notes…………………………………………..........7

5、Production Environment………………………........8

1. Products Parameters

| Parameters |  |
| --- | --- |
| CPU | Rockchip RK3399 (28nm HKMG processing） 6 Core ARM® 64bit ，frequency up to 2.0GHz Big.Little architecture: Dual Cortex-A72 + Quad Cortex-A53, 64-bit CPU |
| GPU | Mali-T864 GPU, OpenGL ES1.1/2.0/3.0/3.1, OpenVG1.1, OpenCL, DX11; Supports AFBC (ARM Frame Buffer Compression) |
| Video Decode | 4K VP9 and 4K 10bits H265/H264 video decoders, up to 60fps 1080P other video decoders (VC-1, MPEG-1/2/4, VP8) 1080P video encoders for H.264 and VP8 Video post processor: de-interlace, de-noise, enhancement for edge/detail/color |
| Power management | RK808 PMU Chip |
| Audio Format | Support MP3/AAC/WAV/WMA/Dolby True HD/DTS-HD/LPCM /H264/VP8/MVC/Double track 4Ω/3W、8Ω/2W output |
| Video format | Supports RMVB,FLV,MPEG1/2/4,AVI,WMV,MOV,MP4,TS,etc |
| Photo format | Supports JPG/BMP/GIF/TIFF/PNG |
| DDR | Dual channel DDR3-1866/DDR3L-1866/LPDDR3-1866/LPDDR4 |
| Flash | High speed eMMC 16G/32G（ optional ） Support eMMC 5.1 with HS400, SDIO 3.0 with HS200 |
| Display Resolution | Max up to 3840X2160 resolution ratio,and HDMI2.0 |
| OS | Android 7.1 |
| Hardware Futures |  |
| Ethernet | 10/100/1000Mbps Ethernet ( Realtek RTL8211E ) |
| Wifi | WiFi module:（AP6356S） 2.4GHz/5GHzdual band WiFi Supports 802.11a/b/g/n/ac protocol 、2x2 MIMO technology Bluetooth 4.1（supports BLE） |
| Output | 1 x HDMI 2.0 ( Type-A ), Supports 4K@60fps 1 x DP 1.2 (DisplayPort) , Supports 4K@60fps 1 x MIPI , Supports Dual channels 2560x1600@60fps 1 x eDP 1.3 ( 4 lanes with 10.8Gbps ) |
| Audio | 1 x HDMI or 1 x DP ( DispalyPort ) , Audio output 1 x Headset , Audio input/output 1 x LINEOUT , Audio output 1 x SPEAKER , 3G/4GCommunicate speaker output ( 1.8W 8Ω/3.1W 4Ω ) 2 x SPEAKER , Speaker output ( 1.5W 8Ω/2.5W 4Ω ) 1 x SPDIF Digital audio ports，used for audio output 1 x 3G/4Gconmunicate by microphone 1 x Microphone，on-board audio input 1 x I2S , support 8 channels |
| USB ports | 4 x USB2.0 HOST，1 x USB3.0 HOST，1 x USB3.0 Type-C |
| Serial Ports | 1 x debug serial Ports（2pin 2.54mm DIP socket），used for developing and debugging |
| Camera | 2 x MIPI-CSI Camera port ( Max up to single 13Mpixel Or Dual 8Mpixel ) 1 x DVP Camera port（Max up to 5Mpixel） |
| PCIe | 1 x Mini PCIe，for LTE |
| SIM | 1 x SIM slot，use for Mini PCIe extend LTE module |
| Touch screen | 1X Support I2C touch pad |
| LED | 1 x Power state LED (Red) ，1 x user defined LED (Blue) |
| Infrared | 1 x IR receiver ，Supports IR remote controller |
| RTC | Support RTC，on-board standby battery port |
| Button | 1 x Reset ，1 x Power，1 x upgrade |
| Power | 1 x DC12V - 3A ( DC 5.5*2.1mm ) 1 x 2pin 2.54mm DIP Socket |
| Reserved Port | 42pin 2.54mm pin： 1 x I2S、2 x ADC、2 x I2C、1 x SPI/UART、2 x GPIO、1 x LINEOUT、1 x SPEAKER |
| System Upgrade | USB Upgrade 、TF Upgrade |

2.Product Picture

1）Port description(only for reference,please make the object as the standard)：

1.RK3399 Soc                                 2.4G LPDDR3

3.32G EMMC                                   4.AP6356S

5.ALC5640 Codec                              6.PMU RK808

7.RTL8211E PHY                               8. H5009NL

9.GL852G USB HUB

串口*8

SATA

单8 LVDS

FFC

双8LVDS

HDMI

1GDDR3

4G NAND

WM8880 CPU

Mini PCI-E 3G

音频

USB*4

摄像头

TF卡座

RJ45

USB*2

音频

VGA

遥控座

i2c

背光

2.5mm DC座

SATA      电源接口

RTC

屏电压跳线

2）PCB Size：(127 * 95.5)mm

3、Interface Specification

1）DCIN (DIP2P-2.54mm)  J5 :

| NO. | Name | attribute | description |
| --- | --- | --- | --- |
| 1 | 12VIN | input | 12V input |
| 2 | GND | GND | GND |

2）42P GPIO Header(DIP 21pin*2*2.5mm)  J8903 :

| NO. | Name | Attribute | Description |
| --- | --- | --- | --- |
| 1 | +12V | power | 12V output |
| 2 | GND | GND | GND |
| 3 | +5V | output | 12V output |
| 4 | GND | GND | GND |
| 5 | +5V | power | 5V output |
| 6 | GND | GND | GND |
| 7 | +3.3V | power | 3.3V output |
| 8 | GND | GND | GND |
| 9 | ADC_IN0 | Input/output | ADC Input/output |
| 10 | ADC_IN2 | Input/output | ADC Input/output |
| 11 | GND | GND | GND |
| 12 | I2S_CLK | Signal wire | Signal wire |
| 13 | I2S0_SCLK | Signal wire | Signal wire |
| 14 | I2S0_LRCK_RX | Signal wire | Signal wire |
| 15 | I2S0_LRCK_TX | Signal wire | Signal wire |
| 16 | I2S0_SDI0 | Signal wire | Signal wire |
| 17 | I2S0_SDO0 | Signal wire | Signal wire |
| 18 | I2S0_SDO1 | Signal wire | Signal wire |
| 19 | I2S0_SDO2 |  |  |
| 20 | I2S0_SDO3 | Signal wire | Signal wire |
| 21 | GND | GND | GND |
| 22 | GND | GND | GND |
| 23 | I2C1_SDA | Signal wire | Signal wire |
| 24 | I2C1_SCL | Signal wire | Signal wire |
| 25 | I2C4_SDA | Signal wire | Signal wire |
| 26 | I2C4_SCL | Signal wire | Signal wire |
| 27 | SPI1_CS | Signal wire | Signal wire |
| 28 | SPI1_CLK | Signal wire | Signal wire |
| 29 | SPI1_RXD | Signal wire | Signal wire |
| 30 | SPI1_TXD | Signal wire | Signal wire |
| 31 | TOUCH_RST_L | Signal wire | Signal wire |
| 32 | LCD_RST | Signal wire | Signal wire |
| 33 | LOUTL | output | LINEOUT-L |
| 34 | LOUTR | output | LINEOUT-R |
| 35 | GND | GND | GND |
| 36 | GND | GND | GND |
| 37 | SPK_LP | output | SPK-LP out |
| 38 | SPK_LN | output | SPK-LN out |
| 39 | GND | GND | GND |
| 40 | GND | GND | GND |
| 41 | SPK_RP | output | SPK-RP out |
| 42 | SPK_RN | output | SPK-RN out |

3）eDP Panel(IPEX-0.5mm 30P)  J6 ：

| NO. | name | Attribute | description |
| --- | --- | --- | --- |
| 1 | VCC -inverter | power | Back Light inverter power input（+5V/+12V） |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 | GND | GND | GND |
| 5 | I2C_ID1 | Signal wire | Signal wire |
| 6 | +3.3V | power | Panel power input |
| 7 | I2C4_SDA_TOUCH | output | TP _I2C_SDA |
| 8 | I2C4_SCL_TOUCH | output | TP _I2C_SCL |
| 9 | LCD_EN_H | output | Lcd enable |
| 10 | TOUCH_INT_L | output | TP _INT |
| 11 | BL_EN | output | Backlight enable |
| 12 | BL_PWM1 | output | Backlight Pwm |
| 13 | LCD_RST | output | LCD_Reset |
| 14 | TOUCH_RST_L | output e | TP _Reset |
| 15 | GND | GND | GND |
| 16 | EDP_TX3P | output | Positive eDP Differential Data Input – 4thChanne |
| 17 | EDP_TX3N | output | Negative eDP Differential Data Input – 4th Channel |
| 18 | GND | GND | GND |
| 19 | EDP_TX2P | output | Positive eDP Differential Data Input – 3rd Channel |
| 20 | EDP_TX2N | output | Negative eDP Differential Data Input – 3rd Channe |
| 21 | GND | GND | GND |
| 22 | EDP_TX1P | output | Positive eDP Differential Data Input – 2nd Channel |
| 23 | EDP_TX1N | output | Negative eDP Differential Data Input – 2nd Channel |
| 24 | GND | GND | GND |
| 25 | EDP_TX0P | output | Positive eDP Differential Data Input – 1st Channel |
| 26 | EDP_TX0N | output | Negative eDP Differential Data Input – 1st Channel |
| 27 | GND | GND | GND |
| 28 | EDP_AUXP | output | DP Aux channel [Positive] |
| 29 | EDP_AUXN | output | DP Aux channel [Negative] |
| 30 | GND | GND | GND |

4）MIPI Panel(FPC-0.5mm 30P)  J8904 ：

| NO. | name | attribute | description |
| --- | --- | --- | --- |
| 1 | MIPI_TX0_D0P | Output | Positive Transmission Data of Pixel 0(Odd) |
| 2 | MIPI_TX0_D0N | Output | Negative Transmission Data of Pixel 0(Odd) |
| 3 | GND | GND | GND |
| 4 | MIPI_TX0_D1P | Output | Positive Transmission Data of Pixel 1(Odd) |
| 5 | MIPI_TX0_D1N | Output | Negative Transmission Data of Pixel 1(Odd) |
| 6 | GND | GND | GND |
| 7 | MIPI_TX0_CLKP | Output | MIPI MDP clock output |
| 8 | MIPI_TX0_CLKN | Output | MIPI MDN clock output |
| 9 | GND | GND | GND |
| 10 | MIPI_TX0_D2P | Output | Positive Transmission Data of Pixel 2(Odd) |
| 11 | MIPI_TX0_D2N | Output | Negative Transmission Data of Pixel 2(Odd) |
| 12 | GND | GND | GND |
| 13 | MIPI_TX0_D3P | Output | Positive Transmission Data of Pixel 3(Odd) |
| 14 | MIPI_TX0_D3N | Output | Negative Transmission Data of Pixel 3(Odd) |
| 15 | GND | GND | GND |
| 16 | LCD_EN_H | Output | Lcd enable |
| 17 | BL_EN | Output | Backlight enable |
| 18 | LCD_RST | Output | LCD_Reset |
| 19 | BL_PWM0 | Output | Backlight Pwm |
| 20 | +3.3V | Output | LCD Power |
| 21 | +3.3V | Output | LCD Power |
| 22 | TP_RST | Output | TP_Reset |
| 23 | TOUCH_INT_L | Output | TP_INT |
| 24 | I2C4_SDA_TOUCH | Output | TP _I2C_SDA |
| 25 | I2C4_SCL_TOUCH | Output | TP _I2C_SCL |
| 26 | +3.3V | Output | TP Power |
| 27 | +3.3V | Output | TP Power |
| 28 | +5V | Output | Back Light inverter power inpu |
| 29 | +5V | Output | Back Light inverter power inpu |
| 30 | +5V | Output | Back Light inverter power inpu |

5）Mipi camera(FPC-1.0mm 30P)  J4601 ：

| NO. | Name | attribute | description |
| --- | --- | --- | --- |
| 1 | NC |  |  |
| 2 | VCC2V8_DVP | Output | VCC2.8V |
| 3 | DVDD_1V2 | Output | VDD1.2V |
| 4 | VCC18_MIPI | Output | VDD1.8V |
| 5 | VCC5V0_SYS | NC |  |
| 6 | GND | GND | GND |
| 7 | AVDD2V8_DVP | Output | VDD2.8V |
| 8 | GND | GND | GND |
| 9 | I2C1_SDA_CAM | Output | I2c date line |
| 10 | I2C1_SCL_CAM | Output | I2c clock line |
| 11 | MIPI_RST0 | Output | Reset line |
| 12 | DVP_PDN0_H | Output | Camera Power EN line |
| 13 | GND | GND | GND |
| 14 | MIPI_MCLK0 | Output | Camera clock outpu10VCCt |
| 15 | GND | GND | GND |
| 16 | MIPI_RX0_D3P | Output | Positive Transmission Data of Pixel 3(Odd) |
| 17 | MIPI_RX0_D3N | Output | Negative Transmission Data of Pixel 3(Odd) |
| 18 | GND | GND | GND |
| 19 | MIPI_RX0_D2P | Output | Positive Transmission Data of Pixel 2(Odd) |
| 20 | MIPI_RX0_D2N | Output | Negative Transmission Data of Pixel 2(Odd) |
| 21 | GND | GND | GND |
| 22 | MIPI_RX0_D1P | Output | Positive Transmission Data of Pixel 1(Odd) |
| 23 | MIPI_RX0_D1N | Output | Negative Transmission Data of Pixel 1(Odd) |
| 24 | GND |  | GND |
| 25 | MIPI_RX0_CLKP | Output | Mipi camera Dp clock |
| 26 | MIPI_RX0_CLKN | Output | MIpi camera DN clock |
| 27 | GND | GND | GND |
| 28 | MIPI_RX0_D0P | Output | Positive Transmission Data of Pixel 0(Odd) |
| 29 | MIPI_RX0_D0N | Output | Negative Transmission Data of Pixel 0(Odd) |
| 30 | GND | GND | GND |

6）Mipi camera(FPC-1.0mm 30P)  J4602 ：

| NO. | Name | Attribute | description |
| --- | --- | --- | --- |
| 1 | NC |  |  |
| 2 | VCC2V8_DVP | Output | VCC2.8V |
| 3 | DVDD_1V2 | Output | VDD1.2V |
| 4 | VCC18_MIPI | Output | VDD1.8V |
| 5 | VCC5V0_SYS | NC |  |
| 6 | GND | GND | GND |
| 7 | AVDD2V8_DVP | Output | VDD2.8V |
| 8 | GND | GND | GND |
| 9 | I2C1_SDA_CAM | Output | I2c date line |
| 10 | I2C1_SCL_CAM | Output | I2c clock line |
| 11 | MIPI_RST1 | Output | Reset line |
| 12 | DVP_PDN1_H | Output | Camera Power EN line |
| 13 | GND | GND | GND |
| 14 | MIPI_MCLK1 | Output | Camera clock outpu10VCCt |
| 15 | GND | GND | GND |
| 16 | MIPI_TX1/RX1_D3P | Output | Positive Transmission Data of Pixel 3(Odd) |
| 17 | MIPI_TX1/RX1_D3N | Output | Negative Transmission Data of Pixel 3(Odd) |
| 18 | GND | GND | GND |
| 19 | MIPI_TX1/RX1_D2P | Output | Positive Transmission Data of Pixel 1(Odd) |
| 20 | MIPI_TX1/RX1_D2N | Output | Negative Transmission Data of Pixel 2(Odd) |
| 21 | GND | GND | GND |
| 22 | MIPI_TX1/RX1_D1P | Output | Positive Transmission Data of Pixel 1(Odd) |
| 23 | MIPI_TX1/RX1_D1N | Output | Negative Transmission Data of Pixel 0(Odd) |
| 24 | GND | GND | GND |
| 25 | MIPI_TX1/RX1_CLKP | Output | Mipi camera Dp clock |
| 26 | MIPI_TX1/RX1_CLKN | Output | MIpi camera DN clock |
| 27 | GND | GND | GND |
| 28 | MIPI_TX1/RX1_D0P | Output | Positive Transmission Data of Pixel 0(Odd) |
| 29 | MIPI_TX1/RX1_D0N | Output | Negative Transmission Data of Pixel 0(Odd) |
| 30 | GND | GND | GND |

7）DVP(FPC-0.5mm 24P)  J4501 ：

| NO. | Name | Attribute | Description |
| --- | --- | --- | --- |
| 1 | DVP_PDN1_H | output | Camera PDNW1 line |
| 2 | GND | GND | GND |
| 3 | I2C1_SDA_CAM | output | I2c date line |
| 4 | VCC2V8_DVP - | output | VCC2.8V |
| 5 | I2C1_SCL_CAM | output | I2c clock line |
| 6 | CIF_RST | output | Reset line |
| 7 | CIF_VSYNC | output | Vsync line |
| 8 | DVP_PDN0_H | output | Camera PDNW0 line |
| 9 | CIF_HREF | output | Hsync line |
| 10 | DVP_DVDD | output | VDD1.8V |
| 11 | VCC1V8_DVP | output | VCC1.8V |
| 12 | CIF_D7 | output | Camera parallel port signal D7 |
| 13 | CIF_CLKOUT | output | Camera CLKIN |
| 14 | CIF_D6 | output | Camera parallel port signal D6 |
| 15 | GND | GND | GND |
| 16 | CIF_D5 | output | Camera parallel port signal D5 |
| 17 | CIF_CLKIN | input | Camera CLKOUT |
| 18 | CIF_D4 | output | Camera parallel port signal D4 |
| 19 | CIF_D0 | output | Camera parallel port signal D0 |
| 20 | CIF_D3 | output | Camera parallel port signal D3 |
| 21 | CIF_D1 | output | Camera parallel port signal D1 |
| 22 | CIF_D2 | output | Camera parallel port signal D2 |
| 23 | GND | GND | GND |
| 24 | NC |  |  |

8）Debug Serial(DIP-2.54mm 3P)  2J2 ：

| NO. | Name | Attribute | Description |
| --- | --- | --- | --- |
| 1 | UART2_RXD | input | RX |
| 2 | UART2_TXD | output | TX |
| 3 | GND | GND | GND |

9）3G/4G SPK (DIP-2.0mm 2P)  J7 ：

| NO. | Name | Attribute | Description |
| --- | --- | --- | --- |
| 1 | SPK_N | output | SPK_N output |
| 2 | SPK_P | output | SPK_P output |

4．Operation Notes：

A, Relative Temperature ≦80%；

B, Storage temperature：-10℃—60℃；

C, Operating temperature：0℃—40℃；

D, Make sure of the electrostatic prevention during the transportation and complete machine assembling

F, No hard pressing, twisting or dropping

G, No power before connecting the panel correctly.

H, Don’t put together with the ones which are electric conduction during working.

I, Don’t repair or dismantle if not professional ones.

.Production  Environment